|  | FORM SRL-CR-075 |
| --- | --- |

Forklift and MHE Pre-start Checklist

| Document number | SRL-CR-075 | Version | 1.0 |
| --- | --- | --- | --- |
| Document owner | Priya Sharma, National WHS Manager | Effective date | 17 November 2025 |
| Approved by | Priya Sharma, National WHS Manager | Next review | November 2027 |
| Applies to | All SteelRiver sites: forklifts, reach trucks, electric pallet jacks, walkie stackers | Status | Controlled. Uncontrolled when printed |

How to use this checklist - Complete one checklist for each item of plant at the start of each shift, before first use (SRL-CR-049). - Tick OK, Defect or N/A for every item. Do not leave an item blank. - Items marked C are critical. A defect in any C item, including a light or alarm that works only some of the time, means the plant is tagged out and not used until it is repaired. - Leave the checklist in the folder for that plant at the forklift parking bay. Supervisors review and sign it before the end of the shift.

| 1 | PLANT AND OPERATOR |
| --- | --- |

| Date |  |
| --- | --- |
| Shift (morning, afternoon, overnight, weekend) |  |
| Time of check |  |
| Asset ID (for example FL-01, RT-02, PJ-01) |  |
| Hour meter reading |  |
| Operator name (print) |  |
| Licence class held (LF) or MHE competency |  |

| A | VISUAL CHECKS: ALL PLANT (KEY OFF) |
| --- | --- |

| Item | C | OK | Defect | N/A |
| --- | --- | --- | --- | --- |
| Forks: no cracks, bends or wear at the heel; locking pins in place | C | ☐ | ☐ | ☐ |
| Mast, chains and carriage: no damage, chains evenly tensioned, no broken links | C | ☐ | ☐ | ☐ |
| Hydraulic hoses and cylinders: no leaks or damage; no oil under the plant | C | ☐ | ☐ | ☐ |
| Tyres and wheels: no cuts, chunks or flat spots; wheel nuts in place | C | ☐ | ☐ | ☐ |
| Overhead guard and load backrest: in place, not damaged | C | ☐ | ☐ | ☐ |
| Data plate (capacity) fitted and readable | C | ☐ | ☐ | ☐ |
| Seat and seatbelt: seatbelt not frayed, buckle works | C | ☐ | ☐ | ☐ |
| Mirrors clean and not damaged |  | ☐ | ☐ | ☐ |
| Fire extinguisher (where fitted) present and in date |  | ☐ | ☐ | ☐ |
| Cabin and steps clean, no loose items on the floor |  | ☐ | ☐ | ☐ |

| B | LPG FORKLIFTS ONLY |
| --- | --- |

| Item | C | OK | Defect | N/A |
| --- | --- | --- | --- | --- |
| LPG cylinder secured in its bracket, valve and hose not damaged | C | ☐ | ☐ | ☐ |
| No smell of gas; no hissing at the cylinder or connector | C | ☐ | ☐ | ☐ |
| Engine oil and coolant levels |  | ☐ | ☐ | ☐ |

| C | ELECTRIC PLANT ONLY |
| --- | --- |

| Item | C | OK | Defect | N/A |
| --- | --- | --- | --- | --- |
| Battery: case not cracked, no leaking electrolyte, cell caps in place | C | ☐ | ☐ | ☐ |
| Battery connector and cables not damaged; battery restrained | C | ☐ | ☐ | ☐ |
| Battery charge indicator: enough charge for the shift |  | ☐ | ☐ | ☐ |

| D | REACH TRUCKS ONLY |
| --- | --- |

| Item | C | OK | Defect | N/A |
| --- | --- | --- | --- | --- |
| Reach mechanism: extends and retracts smoothly | C | ☐ | ☐ | ☐ |
| Operator platform and dead-man pedal work | C | ☐ | ☐ | ☐ |

| E | OPERATIONAL CHECKS: KEY ON, IN A CLEAR AREA |
| --- | --- |

| Item | C | OK | Defect | N/A |
| --- | --- | --- | --- | --- |
| Service brake: stops the plant smoothly | C | ☐ | ☐ | ☐ |
| Park brake: holds the plant | C | ☐ | ☐ | ☐ |
| Steering: smooth, no excess play | C | ☐ | ☐ | ☐ |
| Horn works | C | ☐ | ☐ | ☐ |
| Headlights and work lights work | C | ☐ | ☐ | ☐ |
| Reversing alarm sounds when reverse is selected | C | ☐ | ☐ | ☐ |
| Seat switch: plant will not drive when the operator leaves the seat | C | ☐ | ☐ | ☐ |
| Hydraulic controls: lift, lower, tilt and side shift work smoothly | C | ☐ | ☐ | ☐ |
| Gauges and warning lights on the dash: no fault warnings | C | ☐ | ☐ | ☐ |
| Pallet jack and walkie stacker: belly button reverse stops and reverses the unit | C | ☐ | ☐ | ☐ |

| F | DEFECTS FOUND |
| --- | --- |

Describe each defect: what, where and whether it is constant or comes and goes.

| Reported to (supervisor) and time |  |
| --- | --- |
| Operator signature |  |

| G | SUPERVISOR DECISION |
| --- | --- |

A defect in a C item cannot be classed "OK to use". Tag out, remove the key and raise a Limble work order (priority High).

| ☐ No defects: OK to use | ☐ Critical (C) defect: TAGGED OUT | ☐ Non-critical defect only: OK to use until repaired |
| --- | --- | --- |

| Limble work order number and priority |  |
| --- | --- |
| Next shift told (name and time) |  |
| Supervisor name, signature and date |  |
| Entered in MyOSH by (name and date) |  |

Document history

| Version | Date | Summary of change | Approved by |
| --- | --- | --- | --- |
| 1.0 | 17 November 2025 | First issue in the national document set, from the WHS-076 and WHS-086 pre-start items. Critical items marked C. | Priya Sharma |